# Case Study on Demonstration of Network Communication Models (OSI / TCP-IP) Using a Network Simulation Tool

# 1. Aim

To demonstrate and analyze the OSI and TCP/IP network communication models using a network simulation tool and understand how data is transmitted between devices across a network.

# 2. Objectives

To study the seven layers of the OSI model.

To understand the four layers of the TCP/IP model.

To demonstrate data communication between network devices.

To observe packet transmission using a network simulation tool.

To understand the functions of network protocols at different layers.

To compare the OSI and TCP/IP models.

# 3. Selected Network Simulation Tool

Cisco Packet Tracer

## Overview

Cisco Packet Tracer is a network simulation tool used to design, configure, and troubleshoot computer networks. It allows users to connect computers, switches, routers, and servers and observe how data packets travel through a network.

## Main Features

Network topology creation.

Configuration of routers and switches.

IP address assignment.

Simulation of data packet transmission.

Observation of network protocols.

Testing connectivity using the ping command.

# 4. Network Setup

## Network Devices Required

Two PCs (PC0 and PC1)

One switch

Ethernet cables

Cisco Packet Tracer software

Network Topology: PC0 → Switch → PC1

## IP Address Configuration

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC0 | 192.168.1.1 | 255.255.255.0 |
| PC1 | 192.168.1.2 | 255.255.255.0 |

Both PCs belong to the same network and can communicate through the switch.

# 5. Network Communication Models

## A. OSI Model

The OSI (Open Systems Interconnection) model consists of seven layers:

1. Physical Layer: Transmits raw bits through cables and physical network connections.

2. Data Link Layer: Transfers frames between devices on the same local network using MAC addresses.

3. Network Layer: Handles logical addressing and routing using IP addresses.

4. Transport Layer: Provides end-to-end transport using protocols such as TCP and UDP.

5. Session Layer: Manages communication sessions between applications.

6. Presentation Layer: Handles data formatting, encoding, compression, and encryption.

7. Application Layer: Provides network services used by applications, such as web and file-transfer services.

## B. TCP/IP Model

The TCP/IP model is commonly represented using four layers:

1. Network Access Layer: Handles local network transmission, framing, and physical access.

2. Internet Layer: Handles IP addressing and packet routing.

3. Transport Layer: Provides communication using TCP or UDP.

4. Application Layer: Supports application protocols such as HTTP, DNS, and FTP.

# 6. Procedure for Demonstration

1. Open Cisco Packet Tracer.

2. Add two PCs and one switch to the workspace.

3. Connect both PCs to the switch using copper straight-through cables.

4. Assign the IP addresses and subnet masks shown in the network setup table.

5. Open the command prompt on PC0.

6. Enter the command ping 192.168.1.2.

7. Observe whether PC1 responds successfully.

8. Switch to Simulation mode.

9. Use the Add Simple PDU tool to send a test packet from PC0 to PC1.

10. Use Capture/Forward to observe the packet moving through the network.

11. Open the packet details to examine the protocol information available at each stage.

12. Compare the observed communication process with the OSI and TCP/IP models.

# 7. Demonstration of Data Transmission

When PC0 sends data to PC1, the data passes through the communication layers.

At the sender: Application Data → Transport Segment → IP Packet → Ethernet Frame → Bits

At the receiver: Bits → Ethernet Frame → IP Packet → Transport Data → Application Data

This process is called encapsulation at the sender and decapsulation at the receiver.

In the simulation, the switch forwards the Ethernet frame toward PC1 based on its MAC address information. Since both PCs belong to the same IP network, a router is not required for this communication.

# 8. Comparison of OSI and TCP/IP Models

| Criteria | OSI Model | TCP/IP Model |
| --- | --- | --- |
| Number of layers | 7 | 4 |
| Type | Reference model | Protocol suite and layered model |
| Developed by | ISO | DARPA research community |
| Application functions | Application, Presentation, Session | Application |
| Network addressing | Network layer | Internet layer |
| Transport protocols | Transport layer | Transport layer |
| Common use | Learning and troubleshooting | Practical Internet communication |

# 9. Results

The network topology was created using Cisco Packet Tracer.

IP addresses were configured on both PCs.

Connectivity was tested using the ping command.

Packet transmission was observed in Simulation mode.

The roles of the OSI and TCP/IP layers were studied.

Encapsulation and decapsulation were understood.

The relationship between network protocols and communication layers was demonstrated.

Note: These are the expected results. Record the actual ping and simulation results after performing the experiment.

# 10. Conclusion

This case study demonstrates network communication models using Cisco Packet Tracer. The OSI model explains communication through seven conceptual layers, while the TCP/IP model describes networking through four layers. By configuring two PCs and observing packet transmission, the experiment helps explain addressing, frame delivery, protocol operation, encapsulation, and decapsulation. Therefore, network simulation is a useful method for understanding and analyzing computer network communication.